TASK 3 – STATIC TIMING ANALYSIS (STA)

8-bit ALU using Yosys, Nangate45 Liberty and OpenSTA

# 1. Objective

The objective of Task 3 is to perform Static Timing Analysis (STA) on the synthesized 8-bit ALU, identify the critical timing path, determine the worst slack, and verify whether the design meets the specified timing requirement.

# 2. Tools and Files Used

| Item | Used |
| --- | --- |
| RTL source | Task2/rtl/alu8bit.v |
| STA netlist | Task3/netlist/alu8bit_nangate45.v |
| Standard-cell library | NangateOpenCellLibrary_typical.lib |
| STA tool | OpenSTA 3.1.0 |
| Timing constraint | Task3/constraints/alu8bit.sdc |

# 3. STA Methodology

The ALU RTL was synthesized and technology-mapped to Nangate45 standard cells. The mapped gate-level netlist was loaded into OpenSTA together with the Nangate45 typical Liberty timing library. A 10 ns virtual clock was used as the timing reference for this combinational ALU. Input and output delays were set to 0 ns and clock uncertainty was set to 0.1 ns.

# 4. SDC Constraints

create_clock -name VCLK -period 10.0
set_input_delay 0.0 -clock VCLK [all_inputs]
set_output_delay 0.0 -clock VCLK [all_outputs]
set_clock_uncertainty 0.1 [get_clocks VCLK]

# 5. STA Result

| Parameter | Result |
| --- | --- |
| Clock period | 10.0000 ns |
| Clock uncertainty | 0.1000 ns |
| Critical path startpoint | A[1] |
| Critical path endpoint | Zero |
| Data arrival time | 0.5001 ns |
| Data required time | 9.9000 ns |
| Worst slack (WNS) | +9.3999 ns |
| Timing status | MET |

# 6. Critical Path

The reported worst maximum-delay path starts at input A[1] and ends at output Zero. The path propagates through the following Nangate45 cells:

XOR2_X1 → AOI21_X1 → XNOR2_X1 → AOI221_X1 → OAI21_X1 → OR4_X1 → OR3_X1 → NOR4_X1 → Zero

# 7. Analysis

The maximum data arrival time on the reported critical path is 0.5001 ns. With a 10 ns timing period and 0.1 ns clock uncertainty, the required arrival time is 9.9000 ns. Therefore, the resulting worst slack is +9.3999 ns.

Since the worst slack is positive, the reported maximum-delay path satisfies the specified timing constraint. No setup timing violation is observed for this analysis.

# 8. Conclusion

Static Timing Analysis was successfully performed on the Nangate45-mapped 8-bit ALU using OpenSTA. The critical path from A[1] to Zero has a maximum delay of 0.5001 ns and a positive worst slack of +9.3999 ns. Hence, the design meets the 10 ns timing requirement under the applied constraints.

# 9. Note on Design Type

The ALU is a purely combinational design and contains no flip-flops or physical clock port. Therefore, this STA uses a virtual clock as a timing reference. Conventional register-to-register setup/hold analysis is not applicable to this standalone ALU.